BUILDING WORKS INSPECTION AND REPORT

on

Plot 4

5, Victoria Crescent,

Horsham,

West Sussex

Background Information

The inspection was carried out on 24 May 2019 at which time the weather was overcast, but dry.

Brief Description (Photo 1 – 3)

The subject property comprises a two-storey semi-detached dwelling with off street parking to side and small garden to rear.  The building is believed to be of conventional construction.

Construction

  Roof – Pitched and gabled roof comprising preformed roof trusses overlaid with a microporous breater membrane and covered in slates.

Half round tiles are provided to the ridge.

  Walls – Cavity brick and block walls finished in face brick rising to tile hangings.

  Windows – Windows are double glazed, UPVC construction. Doors are composite construction or UPVC construction

  Ceilings – lined in plasterboard.

  Internal Walls - .  Internal walls are part solid and part metal partition construction dressed in plasterboard.

	Floors – Upper floors are suspended timber joist.   The ground floor is believed to be suspended concrete block and beam. Floors are finished in carpet or ceramic tiling.

Accommodation

  Ground floor: Hall, Sitting Room, Kitchen Diner, Utility, WC

  First floor: Master bedrooms with ensuite, 2 Further Bedrooms, Bathroom/wc, Landing

Instructions

In accordance with your recent instructions we carried out an inspection of this property to provide you with a Building Works Inspection report. External inspection was undertaken from ground level, surveyor’s 3 metre ladder.   Not all parts of the chimneys or roof coverings were clearly visible.  No practical demonstration of rainwater goods was possible.

The property was unoccupied and unfurnished. However, internal inspection was restricted by floor and wall coverings, fixtures and fittings, and the    like. Elements of the structure normally concealed during construction such as floor voids and other voids and ducts were not exposed.   No access was available behind or underneath bathroom or kitchen fittings.

Owing to the physical limitations upon the inspection as noted above, it is possible that other defects may be present but hidden from view. We have highlighted in the report areas that require further and more extensive examination.

Generally

It must be appreciated from the outset that number 3 (Plot 4) has only recently been constructed being completed in the last month or so. Therefore, as with any new house, faults have not yet had the time to materialize and show themselves. As the house settles down it is likely that some faults and defects will become apparent which were not evident at the time of this inspection.

You can expect, for instance, plaster cracking and shrinkage cracking to occur for up to 2 years, particularly above radiators, windows, doors, etc, which will require filling and decorating from time to time. This list will not be exhaustive, but we have tried to highlight the defects apparent and visible at this time.

This report should be construed as a comment upon the overall current condition of the property and is not an inventory of every single defect, some of which would not significantly affect the value of the property. This report is based upon the condition of the accessible parts of the property at the time of inspection and no liability can be accepted for any deterioration in its condition after this date.

NO         LOCATION AND ITEM                                   DESCRIPTION                                        DEFECT (Photo No)                      REMEDY EXTERNAL OBSERVATIONS

1.       STACK                                        None.

2.       ROOF                                          Double   pitched   and   gabled   returning   over   rear addition and forming valley at abutment to adjacent. Roof  slopes  covered  in  natural  slates.    Angled tiles mechanically affixed to ridge.  Valleys formed with fibre glass liner. Valley gutter lined fibre glass

none

3.       RAINWATER GOODS            PVC  construction.    Half  round  gutters  letting  to round downpipes directly to drains.

none

4.       EXTERNAL WALLS               External   walls   believed   cavity   brick   and   block construction.     Elevations   finished   in   face   brick rising   to   tile   hangings   above   first   floor   level. Extended  reveals  to  first  floor  windows  with  lead aprons  beneath.   Edges  of  metal  lintels  and  weep holes  over  ground  floor  openings.    Ground  floor window    openings    provided    with    stone    sills. Subfloor  vents  2  to  front,  2  to  flank,  5  to  rear. Moulded GRP porch

Front –

Poor repair/mortar snot to RHS

sill (Photo 6)

Flank -

none

Rear -

none

Clean / repair

5.       WINDOWS                                 Double  glazed  UPVC.    Trickle  ventilators.    4  to front.   2  to  flank.   4  to  rear.   Extended reveal and aprons beneath.

Front –

none

Flank

Missing end cap RHS of sill 1st floor  window  (photo  7).  Sill Dirty

Protective   tape   to   1st     floor window frame

Fit  end  cap.  Clean  and polish sill

Remove tape and polish

NO         LOCATION AND ITEM                                   DESCRIPTION                                        DEFECT (Photo No)                      REMEDY

Rear

Protective   tape   to   1st   floor window frame

Remove tape and polish

6.       DOORS                                       Front  door  –  Composite  with  glazed  light,  level access sill.

French doors - double glazed, UPVC.

Face  of  door,  frame  and  sill dirty (Photo 8, 9)

none

Clean and polish.

7.       FASCIAS AND SOFFITS        Barge boards, UPVC.                                                     none

NO         LOCATION AND ITEM                                   DESCRIPTION                                        DEFECT (Photo No)                      REMEDY

8	ROOF SPACE                             Accessible  via  lockable  plastic  hatch  to  ceiling  of landing.

Roof  space  provided  with  light.   Un-boarded.   No ladder.

Roof comprises pre-formed timber roof trusses with diagonal timber wind bracing.  (Photo 10) over laid with microporous membrane

Gable walls and party walls formed in block work. Fibre glass insulation between and over joists.

No key to lock

Loft  hatch  poor  trimmed  out and     plaster     board     ceiling susceptible  to  damage  (Photo

11).

No ladder.

Microporous membrane poorly detailed adjacent valley

Builders      debris      including redundant  /  unused  /  damaged metal  channel  and  fibre  glass insulation   wrapping   lying   in roof space (Photo 12, 13)

Insulation     patchy     between trusses (Photo 14)

Check key

Provide                timber trimmers   to   front   and rear edges of loft hatch

Provide Ladder

Cut     away     excessive membrane

Remove waste and tidy

Lift / relay insulation

NO         LOCATION AND ITEM                                   DESCRIPTION                                     DEFECT (Photo No)                      REMEDY INTERNAL OBSERVATIONS

9        Master Bedroom inc En Suite & Dressing Room

Ceiling                                         Plaster  board.  Emulsion  Finish.  Single  pendant  to Master Bedroom.   3  Down  Lights  & extract  to En Suite.  Single rose to Dressing Room.

Walls                                            Plasterboard. Emulsion finish.   En Suite walls part tiled to shower and wet areas behind WC and hand basin.

Floors                                           Suspended timber.  Carpet to Master Bedroom & Dressing Room, Tiling to En Suite.

Master Bedroom –

Minor      ridge      in      ceiling extending from En Suite wall.

Dressing Room –

Minor  gouge  to  ceiling to  rear of   rose,   300   mm   from   rear wall.(Photo 15)

Master Bedroom –

none

Dressing Room –

Minor    open    board    joint    / vertical fracture to internal wall with Master Bedroomdirectly above shelf (Photo 16)

En Suite

Tiling  dirty  /  some  silicon  on tiles

Master Bedroom

Squeaky boards immediately adjacent En Suite Bathroom.

Dressing Room

none

Bathroom

Rub    back,    fill    and redecorate.

Rub    back,    fill    and redecorate.

Rub    back,    fill    and redecorate.

Clean and polish. Re-secure

NO         LOCATION AND ITEM                                   DESCRIPTION                                     DEFECT (Photo No)                      REMEDY

Variations in grout colour immediately adjacent the Shower (Photo17).]

Rake out & renew

Joinery Elements

Windows;  UPVC  casements  to  Master  Bedroom and Bathroom. Window boards timber.

Doors;  3  panel  faced  doors  in  painted  architraves and linings.  Chrome door furniture.

Painted softwood skirting.

none

Master  Bedroom;  Hall  Door Minor opening architrave mitre joints  inside  face  and  external face (Photo 18)

Door face dirty (photo 19)

Shrinkage        fracturing        at architrave  with  adjacent  airing cupboard wall.

Ensuite Door

Minor      opening      architrave joints    internal    and    external faces.(photo 20)

Dressing Room

Minor    shrinkage    fracturing, mitre    internal    and    external faces.

Minor  shrinkage  fracturing  at architrave,    right    hand    side jamb within airing cupboard at abutment to door lining

No matters to report

Rub    back,    fill    and redecorate

Clean     splash     marks from lower face.

Rub    back,    fill    and redecorate

Rub    back,    fill    and redecorate

Rub    back,    fill    and redecorate

Rub    back,    fill    and redecorate

NO         LOCATION AND ITEM                                   DESCRIPTION                                     DEFECT (Photo No)                      REMEDY

Cupboard shelving Faced boards with chrome rails.     none

Sanitary Fittings                           Glazed & tiled shower with mixer.

none

Radiators                                      Steel radiator with independent control to Bedroom.

Chrome towel warmer to Bathroom.

none

Electrical Fittings                         3 double sockets Master Bedroom.   4 light fittings.

Television socket and television outlet.

none

NO         LOCATION AND ITEM                                   DESCRIPTION                                               DEFECT                                REMEDY

10      Front Left Bedroom

Plaster  board.  Emulsion  Finish.  Single  pendant

Bedroom

none

Walls                                            Plasterboard. Emulsion finish.                                        none

Floor                                             Suspended timber.  Carpet                                              none

Joinery                                          Windows;  UPVC  casements  to Bedroom.  Window boards timber.

Doors;  3  panel  faced  doors  in  painted  architraves and linings.  Chrome door furniture.

Skirting – painted softwood timber.

none

Radiator                                        Steel Radiator.  Thermostatic radiator valve.                 No matters to report.

NO         LOCATION AND ITEM                                   DESCRIPTION                                               DEFECT                                REMEDY

Electrical                                      3 double power sockets.  Television outlet.  Socket.

Light switch.

none

NO         LOCATION AND ITEM                                   DESCRIPTION                                               DEFECT                                REMEDY

11      Front Right Bedroom

Ceiling                                          Plaster  board.  Emulsion  Finish.  Single  pendant  to

Bedroom

none

Walls                                            Plasterboard. Emulsion finish.                                        none

Floor                                             Suspended timber.  Carpet                                              none

Joinery                                          Windows;  UPVC  casements  to Bedroom.  Window boards timber.

Doors;  3  panel  faced  doors  in  painted  architraves and linings.  Chrome door furniture.

Skirting – painted softwood timber.

Scratch to window board	     fill/sand and paint

Radiator                                        Steel radiator. Thermostatic radiator valve.                   No matters to report.

Electrical                                      2   double   sockets,   television   socket   and   outlet.

Light Switch.

No matters to report.

NO         LOCATION AND ITEM                                  DESCRIPTION                                               DEFECT                                REMEDY

12      Bathroom (Photo 21)

Ceiling                                         Plasterboard.  Emulsion  finish.     3  Down  Lights.

Extract.

Central rose.  Loft hatch, smoke detector.

No matters to report

Walls                                            Plasterboard linings. Emulsion finish   Boxing with soil pipe. Tiling to wet areas

Joinery                                          Windows; UPVC casements to Bedroom. Window boards timber.

Doors; 3 panel faced doors in painted architraves and linings.  Chrome door furniture.

Skirting – painted softwood timber.

Minor  shrinkage  fracturing  at junction  boxing  to  right  hand wall.  (Photo 22)

Tiling dirty

No matters to report

Face of door dirty. Shrinkage to architrave mitre joints

Rub     back,     fill,     &

redecorate

Tiling requires cleaning and polishing

Clean paint smudges from face of bottom of door. Rub back & fill architrave joints

Sanitary Fittings                           Acrylic  bath  with  glazed  screen  over.   Mixer  riser handset.    China  basin  with  central  mixer.    China WC.

WC lid/seat loose.   Incorrectly fitted (Photo 23

Refit toilet seat

Radiator                                        Steel radiator.  Thermostatic radiator valve                   none

Electrical Fittings                         1  socket.   3  light  switches.   Extract  switch  above door.

none

NO         LOCATION AND ITEM                                   DESCRIPTION                                               DEFECT                                REMEDY

13      Landing & Stairs

Ceiling                                         Plaster board. Emulsion Finish.                                      none

Walls                                            Plasterboard. Emulsion finish.                                        none

Floor                                            Suspended timber.  Carpet                                              none

Joinery                                        Doors;  3  panel  faced  doors  in  painted  architraves and linings.  Chrome door furniture –

Skirting – painted softwood timber.

See bedrooms

Stairs  -    Timber  construction,  carpeted  with  hand rail.  Timber balustrade

Shrinkage   fracturing   at   stair trimmers    to    plaster     work. (Photo    24).    Various    scuff marks     to     balustrades     and landing newel post

Rub     back,     fill,     &

redecorate

Radiator                                      Steel radiator. Thermostatic radiator valve.                   none

Electrical                                     Double sockets,  Light Switches.                                   none

NO         LOCATION AND ITEM                                   DESCRIPTION                                               DEFECT                                REMEDY

14      KITCHEN DINING AREA (Photo 25 / 26)

Ceiling                                         Painted  plasterboard.    10  down  lighters.    Pendant rose.       Smoke   detector   and   carbon   monoxide detector.

none.

Walls                                            Painted plasterboard linings.                                          none

Floors                                           Suspended concrete / beam and block.  Tiled finish.     none

Joinery                                          Windows; 3 UPVC casements. Window boards timber.

none

Door  – Timber framed; part glazed. Painted architraves and linings.

Skirtings – painted softwood timber.

KITCHEN                                    Wall and counter units with surface.  Built in fridge freezer.    Gas  hob.    Electric  oven.    Extract  unit. Built in dishwasher.

Shrinkage fracturing to architrave at mitre joints and abut to adjacent plasterwork. (Photo 27, 28)

Open joint to rear of island unit.

none

Appliances not tested

Rub back, fill, &

redecorate

Rub back, fill, &

redecorate

Electrical Fittings                         6  Double  outlets.   Switched  outlets  for  appliances

Under counter lighting. Light Switches

NO         LOCATION AND ITEM                                   DESCRIPTION                                               DEFECT                                REMEDY

15      Sitting Room

Ceiling                                         Painted plaster board. Emulsion Finish. Single rose.     none.                     .

Walls                                            Painted plaster board. Emulsion Finish.                         none

Floor                                             Concrete beam and block with carpet finish.                 none

Joinery                                          Door – glazed timber, chrome furniture, painted linings and architrave.

French doors – double glazed, UPVC.

none

No matters to report. Unable to test.

Skirtings – painted softwood.

Open mitre joint to right hand flank wall 1 m back from corner.

Rub back, fill, &

redecorate

Radiator                                        Steel radiator.  Thermostatic radiator valve.                  none

Electrical Fittings                          3 Double, Double television socket outlet.   2 light switches.

none

NO         LOCATION AND ITEM                                    DESCRIPTION                                               DEFECT                               REMEDY

16      UTILITY AREA (Photo 29)

Ceiling                                         Plaster   board.   Emulsion   finish   2   down   lights.

Extract.

None.

Walls                                            Plaster board lining. Emulsion finish                                No matters to report.

Floor                                             Suspended concrete / tiled finish.                                     No matters to report.

Joinery                                          Door – 3 panel.  Painted linings and architraves.

Chrome furniture.

Counter units with stainless steel sink and mixer. Radiator                                        Single radiator.

Shrinkage fracturing to architrave mitres and door linings to left hand side.

Expanded foam filler in cupboard (photo 30)

Rub back, fill, &

redecorate

Cut back and make good expanding foam below sink unit.

Electrical Fittings                         2 double sockets plus spurred outlet light switch and thermostat.

NO         LOCATION AND ITEM                                    DESCRIPTION                                               DEFECT                               REMEDY

17      CLOAKROOM

Ceiling                                         Plaster board. Emulsion finish. Extract. Down lights    No matters to report                      .

Walls                                            Plaster board. Emulsion finish.                                       Minor scuff / gouge marks to soil stack and right-hand wall adjacent and around access panel. (Photo 31)

Rub back, fill, &

redecorate

Floor                                             Suspended concrete / tiled finish.                                   None.

Sanitary Fittings                            China basin and WC.                                                     No matters to report

Joinery                                          Door – 3 panel.  Painted linings and architraves.

Chrome furniture.

No matters to report

Radiator                                        Steel radiator                                                                   No matters to report

NO         LOCATION AND ITEM                                    DESCRIPTION                                               DEFECT                               REMEDY

18      HALLWAY & COAT CUPBOARD

Ceiling                                         Plasterboard ceiling linings. Emulsion finish,  4 down lighters.  Smoke detector.

No matters to report

Walls                                            Plaster board linings. Emulsion finish.                             None.

Floor                                             Tiled finishes.  Concrete beam and block.                        None.

Joinery                                          Windows – double glazed, casement, UPVC. Timber window boards

Doors.

Painted softwood skirtings

None.

See respective rooms

Radiator                                        Single panel radiator.                                                        None.

NO         LOCATION AND ITEM                                   DESCRIPTION                                               DEFECT                                REMEDY

19      Services

Services have not been tested. You should obtain copies of all commissioning and test certificates together with guarantees available

Electricity                                     External meter

Distribution  board  in  Hall  Cupboard  (Photo

32)

Gas                                               External meter

Heating                                         Wall mounted fan flue boiler (Kitchen – Boiler

Valiant Home System 15) serving radiators

Hot water                                     Pressurized unvented in first floor airing cupboard

(Photo 33)

Drains                                           Believed  mains  -  PVC  above  ground  Inspection chambers PVC.

Rain water goods – solicitors to verify if mains or to soakaway

Rims loose                                     Secur

OTHER MATTERS

You are advised to ensure that you ascertain the following documentation and information from the Developers:-

 Is the property fully covered by an NHBC insurance policy or similar against major defects for at least the next ten years?

 Has the developer obtained full planning permission, Building Regulation approval and obtained a Completion

Certificate?

 Will the developer provide you with a full set of drawings and construction details for maintenance purposes?

 Has the roofer provided a guarantee? Again this ideally should be for at least 20 years, labour and materials?

 Is there full electrical certification available.

 Is there a GAS SAFE Certificate available.

 Confirm that the gas central heating boiler has been bench marked.

 Energy Performance Certificate for the property.

 Air Test Performance Certificate.

 Is there a homeowner’s manual on all appliances, this should include all working drawings, services, layout, full plans etc.

 Window and door guarantees and FENSA Certificate.

 Drainage test certificate and confirmation that this is correctly installed and laid to correct falls.

Finally you must appreciate that once you start to live in the property then further defects are likely to become apparent. If you can,

you should ask to retain a small percentage of the purchase price (2.5%) for a defects liability period (6 months or more), which would encourage the developer to come back and complete any outstanding snagging items that become apparent.

24th May 2019

1 (Front)                                                                                           2 (Side)

3 (Rear)                                                                                                 4

5                                                                                                       6

7                                                                                                       8

9                                                                                                      10

11                                                                                                     12

13                                                                                                     14

15                                                                                                     16

17                                                                                                     18

19                                                                                                     20

21                                                                                                     22

23                                                                                     24